Movable sound-insulating partition wall Model: Variflex 88 

Movable partition wall of individually operable elements with a frame construction of torsionally stiff, non-deflecting aluminum and steel profiles.  Clad on both faces with 
E1-grade quality cover boards to DIN, top-hung vibration-free suspension for acoustic isolation, Acoustically isolated vertical profiles to ensure minimal structure-borne sound transmission. Integrated double-chamber sealing rails/lip seals in aluminum finish. Element thickness not to exceed 88 mm. It must be possible to replace the cover panels without having to remove the elements from the track.

To specifically and economically dimension the structurally bearing components to prevent flexure, partition element weights shall not exceed 49 kg/ m2 for a sound reduction index 
Rw = 58 dB, 41 kg/ m2 for 56 dB, 32 kg/ m2 for 53 dB, 24 kg/ m2 for 46 dB and 23 kg/ m2 for 41 dB. Constructions exhibiting higher weights will therefore be rejected.
Sound attenuation of the operable partition elements to be tested to EN 10140-3: 2010 (Annex A: Walls) and be validated by corresponding test certificates/reports.  
Testing and measurement to be conducted in a wall test facility compliant with EN ISO 12999-1: 2014. Calculation of the weighted sound reduction index and the spectrum adaptation terms to be in accordance with EN ISO 717-1: 2013.    



Extendable and retractable sealing rails 

Each element to feature spring-loaded double-chamber sealing rails top and bottom operated by crank winding spindle mechanism (trapezoidal thread) for pressing against floor and ceiling track and automatically compensating for floor unevenness.
Sealing rail end pieces comprised of polyurethane moldings to be fitted to ensure optimum vertical sealing between the extended sealing rails. 
Sealing rails to be primarily of aluminum and designed to meet the highest demands in terms of acoustic performance and stability. 
Semi-automatic actuator function (optional)

As the partition closes or opens, the telescopic element and the horizontal sealing rails of the individual elements are to be automatically extended and retracted by means of an electrically powered motor. Power connection between elements to be made by means of end face plug-in contacts, with the partition connected to the line supply via a ready-plugged power supply unit and a central master switch. 
Function of the electronic overload relay 
For reasons of safety, mechanical limit switches are not permitted.
Emergency unlocking device at ergonomic height to be provided for all elements so that the partition may still be operated in the event of a possible fault (power failure). Power supply to be 41 volts for only minor voltage loss across the circuit, ensuring the function of the sealing rails.  Designs employing lower voltages are not permitted.



Element interconnection

The elements are interconnected by means of positive interlocking concave/convex aluminum profiles with the option of an integrated magnetic strip.
Additional flexible seals are to be provided within the interfacing joint. Mechanically acting element connectors are not permitted.



Closure element

Final partition closure element in each case to be designed as a compensatory telescopic element.

Telescopic element to be constructed from E1-grade quality panels to DIN. Material to be identical with that of the main partition.
Telescopic section with useful extension stroke of up to 120 mm with adjustable contact pressure to ensure the partition is properly closed.


Pass door elements (optional)

If single or double pass door elements are included in the configuration, these are to be equipped with a surrounding portal frame of sturdy steel sections.  Lateral posts to be provided with pressure pads to ensure positive engagement with floor.  Further stability to be provided in the form of locking projections engaging in floor sockets. Door leaf to be provided with steel frame and automatically operable sealing rail. Flexible sealing components in compression contact with ceiling (sealing rail) and floor (sealing feet in door posts)   to be operable from the end face.  

Double pass doors to feature an active leaf and an inactive leaf equipped with an emergency unlocking device.

Acoustically effective surface (optional)


Sound absorption/acoustic construction of the cover panels:

Acoustically effective sound-absorbing cover panel construction comprising a perforated composite surface, acoustic fleece and acoustic core.  The total thickness of the element with an acoustic panel on one side shall not exceed 104 mm, and with acoustic panels on both sides, it shall not exceed 120 mm. 

The surface decor must be provided in the form of a directly coated laminate. Options available from standard in-house collection for acoustic panels: Platinum White (sim. to RAL 9016), Brilliant Grey (sim. to RAL 7024), Anthracite Graphite (sim. to NCS S1002-Y) or Fine Maplewood.   

The formaldehyde content in the drilled state – as measured and verified using the WKI test (Wilhelm-Klauditz-Institut) prescribed in EN 717-1:2005-1 – must be below the permissible threshold of 0.1 ppm according to the EU Chemicals Regulation.   

[  ] 
Microperforation 4/4/1.5

Hole diameter: 1.5 mm 

Spacing (hole center intervals): 4 mm

    
Sound absorption value (per ISO 11654): αW 0.50   

with the following αP values to be achieved at the various frequency ranges indicated:

125 Hz = 0.05 αP; 250 Hz = 0.20 αP; 500 Hz = 0.60 αP; 1000 Hz = 1.00 αP; 
2000 Hz = 0.65 αP; 4000 Hz = 0.40 αP
[  ] 
Perforation 8/8/3

Hole diameter: 3 mm 

Spacing (hole center intervals): 8 mm

    
 Sound absorption value (per ISO 11654): αW 0.45 

with the following αP values to be achieved at the various frequency ranges indicated:

125 Hz = 0.10 αP; 250 Hz = 0.20 αP; 500 Hz = 0.50 αP; 1000 Hz = 0.85 αP; 
2000 Hz = 0.75 αP; 4000 Hz = 0.35 αP
The sound absorption test must be performed in reverberation rooms in accordance with ISO 354:2003,  with the measurement across the acoustic surface being made with the panel placed directly on the reverberation room floor.  Additional means to create clearances from the floor are not permitted. Assessment of the sound absorption to be performed in accordance with ISO 11654 and verified in the form of a certificate included with the quotation/tender specifications. 
Element suspension

Each element to be hung at one or two points from a ceiling-mounted aluminum track and operate on multiple roller carriers; track rollers to be located in ball bearings.  Roller carriers to be secured to partition element by means of roll-joint pins with horizontal ball bearings. No ball races or sliding friction pads/discs permitted.  


  
Element adjustability 

To compensate for minor ceiling sag, each element to be readily adjustable vertically without opening up the ceiling or element. In the event of superficial damage, cover panels to be replaceable without removing or dismantling elements or other components of the construction.


Track system

R-Track
Intersections, T-junctions, other junctions and corners in layout to be provided in the form of closed-die forgings with stabilizing/support roller assemblies.  Rolling friction required at all points of contact to ensure easy transit of elements at intersections and in stacking track/parking zone. 


K-Track
Curves and switches to be fabricated from formed and bent aluminum extrusions.
Mechanical coding to be incorporated in the track system to ensure easy operation of the elements.



Fully automatic sound barrier (optional)


240V/50 Hz supply to be provided by others up to the track intersection for connection of the switch box/control unit for the fully automatic sound barrier.

The ceiling must be provided with an inspection hatch to the side of the track.

Track opening/slot to be automatically closed by a 24V motor (mechanical actuation not permitted).

Emergency crank operation to be available in the event of a power failure.

The sound barrier is activated by means of  a switch and is automatically

tripped in the event of an overload.

Max. mounting height of the fully automatic sound barrier not to exceed 100 mm (including electronic components) – higher sound barriers not permitted.

Track installation

Track systems to be secured by means of adjustable steel suspension/hanger assemblies to load-bearing structural components provided by others (e.g. steel substructures, concrete beams, etc.). Suspension/hanger assemblies to be supplied by bidder.  Adjustability to be ensured to allow compensation of subsequent minor ceiling sag.  Rigid, non-adjustable hanger constructions not permitted.
Materials employed to be provided with corrosion-inhibiting coating/treatment.
Partition systems offered to exclusively feature track constructions and suspension assemblies complying with the definition of “simple steel structures” per DIN 4100 (cf. German VOB [Construction Contract Procedures], Part C, DIN 18335).
Bidder to possess a valid welding fabricator certificate to EN ISO 3834-3.   Copy of this certificate to be attached to the quotation.


Sound baffle (optional)

Double-skin sound baffle (for sound reduction index values up to Rw = 50 dB) or quadruple-skin baffle (Rw = 51 dB to Rw = 58 dB) to be provided by bidder around the track system. Baffle performance to correspond to specified sound reduction value of partition elements.  Baffle to be properly and tightly sealed against the track and the adjacent building components. Cavities between plasterboard skins to be filled with non-trickling mineral wool.  Ceiling joints to be gun-sealed with silicone mastic (permanently elastic type). Aforementioned substructures/assemblies, fills and baffle systems to be included in the unit prices.
Operable partition wall system to be certified as ball-impact-resistant.

An Environmental Product Declaration (EPD) to ISO 14025 must be assigned to the partition wall system.   The life cycle analysis (LCA) must be carried out in accordance with ISO 14040 employing a methodology commensurate with the ecobalance of the system.


General quality management / ISO certificate

The manufacturer of the partition system must have introduced and must operate a registered quality management system to (DIN) (EN) ISO 9001 and be able to confirm this with a valid certificate. 

Item 1
Quantity:
Operable partition system as described above = .... units
Dimensions:
- Clear width = …………… mm
- Clear overhead height = …………… mm 
- Height of suspension = …………… mm
- Partition weight/m² =………… kg
- Element thickness = …………… mm

Operation:
(   ) manual
(   ) semi-automatic

Cover panels / Profiles:
(  ) Type K (cover panel with visible surface edges)
(  ) Type U (cover panel with protective all-round trim)
(  ) Magnetic strip integrated in vertical profile
(  ) With fire protection cover panels of class B-s2-d0, to EN 13501-1




Elements:
- Total number: =……… units Total number of elements:
of which:
- Full wall (VE) =……… units
- Telescopic =……… units
  (  ) External (outboard) telescopic (TE)
- Pass door (DT) =……… units
- Double pass door (DTZ) =……… units.
- Corner (EE) =……… units.


Surface finish:
Classic Collection:
[  ] Melamine Hüppe Collection/Selection:……………

Design Collection:
[  ] Veneer: Wood type …….….......................
[  ] Painted to RAL: Color ….………
Functional Collection:
[  ] Projection screen surface, magnet-attracting:…….
[  ] Magnet-attracting/Writable surface: ..........



Sound insulation requirement:
(  ) 41 dB (23 kg/ m2) with test certificate 
(  ) 46 dB (24 kg/ m2) with test certificate
(  ) 53 dB (32 kg/ m2) with test certificate
(  ) 55 dB (41 kg/ m2) with test certificate 



(  ) 58 dB (49 kg/ m2) with test certificate



Stacking track/parking zone:
- Layout per attached drawing 
Element suspension:
(  ) Single-point suspension; only possible with 90° stacking track; weight-dependent
(  ) Two-point suspension
Track system:
(  ) R-track of aluminum, right-angle junctions - up to 500 kg
(  ) K-track of aluminum, coded track with curves and switches - up to 250 kg

(  ) Fully automatic sound barrier required as per drawing attached   

Make: DORMA Hüppe
Model: Variflex 88 or equivalent
Inclusive unit price includes delivery and 
installation ready for operation.                                 U.P. 
